The Trophic Diatom Index (TDI) and Diatom Quality Index (DQI) - monitoring method

(fact sheet)

| ##### Water Category | River |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Composition and abundance of aquatic flora (phytobenthos) |  |  |
| Name of the method  | The Trophic Diatom Index (TDI) and Diatom Quality Index (DQI) |  |  |
| Method proposed by  | ###### United Kingdom |  |  |
| Method commonly used in  | United Kingdom |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| TDI and DQI are indices of the trophic status and organic loading of rivers, based on diatoms. TDI incorporates a procedure for sampling, analysis and an analytical quality assurance. The DQI differs from the TDI solely in having a similar scale to the MTR (see MTR factsheet). The DQI is 100 – TDI. Sensitivity values (s) have been assigned to each species of diatom that relate to the nutrient concentrations (specifically filtered reactive phosphorus) in which they are most abundant. Values of s are from 1 (low nutrient concentrations) to 5 (high nutrient concentrations), broadly based on the OECD criteria for lakes. Indicator values (v) have been assigned to each species based on the narrowness of their distribution amongst the 5 sensitivity values. Samples are collected by a standard procedure, and 200 diatoms identified. The abundance of each species present is calculated, as a proportion of the 200. The TDI is the abundance-weighted average sensitivity to nutrient status. The tolerance of diatom species to organic pollution have also been determined. The degree of organic pollution is indicated by the proportion of taxa classified as being tolerant to organic pollution. This assessment is made on a four category classification: A tabular matrix is given in the users’ manual to interpret the combination of tolerance to organic pollution and TDI or DQI. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Not reference based |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | No |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | CEN standard for diatom sampling will be prEN 13946 and the standard for lab.stages will be prEN 14407. The former should be published early in 2003, the other in 2004. Both face a formal vote, but I suspect that the changes will be minor. |
| National Standard (yes/no) – if yes provide reference number |  |  | No |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes |
| Reference material |  |  |  |
| Kelly, M.G. & B.A. Whitton, 1995 Use of diatoms to monitor nutrients. NRA R&D Note 431. Bristol: National Rivers Authority. Kelly, M.G., C. Adams, A.C. Graves, J. Jamieson, J. Murray-Bligh, S. Prichard and S. Wilkins, 2001 The Trophic Diatom Index: a users’ manual. R&D Technical Report E2/TR2. Bristol: Environment Agency. Kelly, M.G., 1996 Diatoms as tools for water quality managers. R&D Technical Report E3. Bristol: Environment Agency. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The new TDI method is fully is fully compliant with CEN standards for diatom sampling (prEN 13946) and for laboratory analysis (prEN 14407). However, both are written in fairly general terms and it may be that a WFD-specific method may require tighter specifications in some aspects. Complementary indices may be needed to relate macrophyte species to wider ecological conditions than simply nutrients and organic pollution. Observed indices would need to be related to those in reference conditions. This is the subject of a current research project. Relationship to organic pollution may need to be made more robust. MTR values need to be re-banded to produce a WFD-compliant classification scheme. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Martyn Kelly Jane Jamieson |  |
| Expert e-mail address |  | Bowburn_consultancy@compuserve.com Jane.jamieson@environment-agency.gov.uk |  |
| URL: |  |  |  |
| Telephone number |  | +44 (0)191 377 2077 +44 (0)1491 828 548 |  |