ECN epilithic diatoms (rivers) - monitoring method

(fact sheet)

| ##### Water Category | Rivers |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Composition and abundance of aquatic flora (phytobenthos) |  |  |
| Name of the method  | Protocol for monitoring epilithic diatoms at ECN river sites |  |  |
| Method proposed by  | ###### UK |  |  |
| Method commonly used in  | UK |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Epilithic diatoms are sampled in the river sites of the Environmental Change Network because of their known response to changes in acidification and nutrient status. The measure estimated in running waters will be species composition and relative abundance. |  |  |  |
| Sampling is undertaken at a 50 m stretch of river coincident with that used for ECN macrophyte monitoring. Three sampling locations are identified in this stretch and at each location 5 cobble sized stones are selected and removed. The epilithic diatoms are removed with a toothbrush and rinsed into a tray. After successive brushing the composite sample is homogenised and sub-sampled. This small sub-sample is preserved. Permanent slides are prepared in the laboratory for archiving. Any future analysis of these slides will involve identification of 500 diatom valves to species level with estimates of relative abundance. |  |  |  |
| The data are used for long-term trend analysis to determine impacts from pressures such as climate change. Interpretation of this measure is made with reference to river flow. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| This method is not used to make a judgement on the status of the environment and consequently there is no comparison to a reference value of reference conditions. Comparisons are made primarily with observations at the same sites in previous years. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | No |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | No |
| National Standard (yes/no) – if yes provide reference number |  |  | No |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes - for ECN monitoring network (FDT protocol – see reference material) |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes |
| Reference material |  |  |  |
| Sykes, JM., Lane, AMJ. and George, G. (Eds) (1999). The United Kingdom Environmental Change Network: protocols for standard measurements at freshwater sites. Institute of Terrestrial Ecology, Huntingdon. (ISBN: 1 870393 49 X) |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The method provides a standard technique for estimating epilithic diatoms in rivers. |  |  |  |
| The WFD requires species composition and abundance to be estimated and these measures could be derived using this method. The measures are not compared with reference conditions and there is no classification scheme relating to quality status. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Alastair Ferguson |  |
| Expert e-mail address |  | Alastair.ferguson@environment-agency.gov.uk |  |
| URL: |  | www.environment-agency.gov.uk |  |
| Telephone number |  | 01454 878849 |  |