IBGN-Expert system - monitoring method

Monitoring WG 2.7 - 29/05/2002

| ##### Water Category | River |  |
| --- | --- | --- |
| ##### Water Framework Directive Quality element | Benthic invertebrate fauna – composition and abundance |  |
| Name of the method  | IBGN Expert System |  |
| Method proposed by  | ###### France |  |
| Method commonly used in  | France |  |
| Short description of the method (approx. 10 lines): |  |  |
| A more detailed exploitation of IBGN faunistic list (see ad hoc fact sheet) using new concepts of taxa’s biological and ecological profiles has been proposed by the Metz University to provide a better identification of community stresses with a reasonable sampling and identification effort (IBGN protocol). This system aims in particular to identify the community composition that should be observed in absence of significant anthropogenic stress. The deviation between observed and reference conditions is measured with a similarity index. The system assess the biogenic capacity of in place mesohabitats. It also calculated classical ecological indices related with community structures. The results are expressed either with multi-factors maps or with quality classes when it is relevant. |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |
| Technical report (Water-Agencies) & scientific publication (University of Metz, Ph. Usseglio & J.N. Beisel) in preparation. |  |  |
| Reference conditions compliant with WFD (yes/no) |  | <to be completed> |
| Status of method |  |  |
| International Standard (yes/no) – if yes provide reference number |  | <to be completed> |
| National Standard (yes/no) – if yes provide reference number |  | <to be completed> |
| National method (yes/no) – if yes provide reference number and details |  | <to be completed> |
| Published in literature (yes/no) – if yes provide reference details |  | <to be completed> |
| Reference material |  |  |
| Applicability of the method to WFD (Expert’s assessment of method): |  |  |
| With expert system, the IBGN method become very consistent with WFD as it allows assessment related to reference condition with no or very few human pressure. |  |  |

| Contact details |  |
| --- | --- |
| Name of contact/expert | Stephane Stroffek |
| Expert e-mail address | Stephane.stroffek@eaurmc.fr |
| URL: |  |
| Telephone number | 33 4 72 71 26 03 |