Acidification index - monitoring method

(fact sheet)

| ##### Water Category | Rivers |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Composition and abundance of benthic invertebrate fauna |  |  |
| Name of the method  | Acidification index |  |  |
| Method proposed by  | ###### United Kingdom |  |  |
| Method commonly used in  | United Kingdom (Wales) |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| This is a biological classification scheme for sensitivity to acidification. It is more correctly an index of acid conditions. The index recognises four categories of acidification. A flow chart is provided in the paper to enable the category to be determined. The classification scheme was devised from national data. The classification is for base poor streams only, where hardness < 30 mg.l-1 as CaCO3, alkalinity < 25 mg.l-1, or conductivity < 20 S.cm-1. Samples are to be collected from November to April with a 1 mm pond-net from shallow, fast-flowing riffles. A 1 minute kick sample is recommended. The classification scheme and its application nationally is being investigated in an Environment Agency R&D Project P2-090 Acid waters monitoring - indicator populations. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Not reference-based. A RiVPACS-compatible index is under development, which will allow reference conditions to be determined by reference to RIVPACS reference samples. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | No. A compliant system is under development. |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | No |
| National Standard (yes/no) – if yes provide reference number |  |  | No |
| National method (yes/no) – if yes provide reference number and details |  |  | No |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes |
| Reference material |  |  |  |
| Rutt, G.P., N.S. Weatherly and S.J. Ormerod, 1989. A macroinvertebrate indicator system for the detection of acidic conditions in upland streams throughout Britain: test version. University of Wales College of Cardiff report to Nature Conservancy Council. NCC Contract No. HF3-03-408. Rutt, G.P., N.S. Weatherly & S.J. Ormerod, 1990. Relationships between physicochemistry and macroinvertebrates in British upland streams: the development of modelling and indicator systems for predicting stream fauna and detecting acidity. Freshwater Ecology, 24 : 463 - 480. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| This index could be used as one of a number of classification metrics or as part of diagnostics to help determine causes of poor ecological quality and hence apppropriate programmes of measures. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Graham Rutt |  |
| Expert e-mail address |  | Graham.rutt@environment-agency.gov.uk |  |
| URL: |  |  |  |
| Telephone number |  | +44 (0)1554 757 031 |  |