Swedish fish index - monitoring method

(fact sheet)

| ##### Water Category | River |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Fish – species composition, abundance and age structure |  |  |
| Name of the method  | Swedish fish index |  |  |
| Method proposed by  | ###### Sweden |  |  |
| Method commonly used in  | Sweden |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| The fish index was developed as a part of the national criteria for assessment of environmental quality. It can be applied with test-fishing data from a single locality and single occasion in streams and rivers using the the standard method « Quantitative sampling of fish with electricity » together with information on stream width, stream velocity, catchment area, proportion of upstream lakes of the catchment area and altitude. Presently, it includes seven metrics, i.e. five indicators of the structure and function of the fish community and two indicators of environmental disturbance. For each metric the deviation between observed and expected values is scored from 1 (none or minor deviation from reference values) to 5 (very large deviation from reference values). The mean score of all metrics is used to calculate a final index. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Metrics and their reference values were derived using the Swedish Electrofishing RegiSter (SERS), including data collected by using the standard method ”Quantitative sampling of fish with electricity”. At present the database contains more than 22 000 electrofishing occassions and 9 000 fishing sites (www.fiskeriverket.se). The database consist of five ”files”: LOCALITIES, contains geographical data for the fishing sites, FISHING, contains data of the electrofishing performance, CATCH, contains data about the catch; species occurrence and abundances, LENGTH, contains all the fish lengths from the catch, CHEMISTRY, contains data of the water chemistry (if such a sample has been taken). The files are connected with the x- and y-coordinate (Swedish coordinate system, RAK). The data in the database will be used in the revision of the Swedish fish index, including values for type specific reference states. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | Not exactly, but the Swedish fish index will be revised |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | No |
| National Standard (yes/no) – if yes provide reference number |  |  | Not exactly |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes, see “reference material” |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes, see “reference material” |
| Reference material |  |  |  |
| Appelberg, M., B. Bergquist & E. Degerman. 1999. Fisk . p. 167-239, In: Naturvårdsverket, Bedömningsgrunder för miljökvalitet. Sjöar och vattendrag. Bakgrundsrapport 2. Biologiska parametrar. Naturvårdsverket Rapport 4921 (In Swedish). |  |  |  |
| Appelberg, M., B.C. Bergquist & E. Degerman. 2000. Using fish to assess environmental disturbance of Swedish lakes and streams – a preliminary approach. Verh. Internat. Verein. Limnol. 27 : 311-315. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The presented metrics of the fish index are more or less associated with species composition, abundance and age structure, but the index has to be revised in terms of both metrics and their type-specifice reference values to get full applicability to WFD. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Björn Bergquist |  |
| Expert e-mail address |  | bjorn.bergquist@fiskeriverket.se |  |
| URL: |  |  |  |
| Telephone number |  | +46 8 6200445 |  |