IBD - monitoring method

(fact sheet)

| ##### Element of WFD  | Aquatic flora – Phytobenthos composition and abundance |
| --- | --- |
| Name of the method  | IBD (Indice biologique diatomées) |
| Method proposed by  | ###### France |
| Method commonly used in  | France since 1996 |
| Short description of the method : |  |
| This method aims the monitoring of general water quality (organic matter, nutrients, chlorides, pH….) and is based on ecological profiles of 209 diatom taxa identified from a large data set (more than 1330 sites covering the whole France with associated water chemistry – 14 chemical parameters). The different steps of this method are fully described both in the French standard and in the user guide (see references below). diatoms are sampled by scraping natural substrata (rocks, pebbles..) or artificial surfaces (bridges, walls…) or macrophytes or artificial substrates (glass…..), diatoms are then cleaned using oxidants before mounting. 400 units are counted and only those belonging to the IDB taxa list are identified. All taxa which aboundance is below 7.5 per mile are eliminated as well as taxa which aboundance are below the threshold of ecological significance (one threshold par IBD taxon), the calculation is made by integrating the ecological profiles of the remaining taxa, each one being weighted according to its aboundance, the final score is transformed into a score on a scale from 0 to 20 to allow comparisons with other biological indices. |  |
| References : |  |
| - AFNOR (2000) Norme NF T 90-354 : Qualité de l’eau – Détermination de l’Indice Biologique Diatomées (IBD), juin 2000, 63 pp. |  |
| - Prygiel J, Coste M (2000) Guide méthodologique pour la mise en œuvre de l'indice Biologique Diatomées NF T 90-354. Etude Agences de l'Eau-Cemagref Bordeaux, mars 2000, Agences de l'Eau, 134 pp. + clés de détermination (90 planches couleurs) + cdrom bilingue français-anglais (Tax'IBD). |  |
| - Prygiel J., Carpentier P., Almeida S., Coste M., Druart J.-C., Ector L., Guillard D., Honoré M.-A., Iserentant R., Ledeganck P., Lalanne-Cassou C., Lesniak C., Mercier I., Moncaut P., Nazart M., Nouchet N., Peres F., Peeters V., Rimet F., Rumeau A., Sabater S., Straub F., Torrisi M., Tudesque L., Van de Vijver B., Vidal H., Vizinet J., Zydek N., 2002. Determination of the biological diatom index (IBD NF T 90 - 354): results of an intercomparison exercise. J. applied Phycology 14 (1): 27-39. |  |
| - Prygiel J., 2002. Management of the diatom monitoring network in France. J. applied Phycology 14 (1): 19-26. |  |
| Applicability of the method to WFD : |  |
| With expert system, the IBD method is fairly consistant with WFD, but the assessment is not completely related to reference condition with no or very few human pressure. |  |

| Expert e-mail adress | j.prygiel@eau-artois-picardie.fr |
| --- | --- |

06/06/2002