- monitoring method

(fact sheet)

| ##### Water Category | River |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Benthic invertebrate fauna- Composition and abundance + habitat survey +physicochemical parameters |  |  |
| Name of the method  | HBMWP (Hellenic BMWP) +HASPT+Hindex |  |  |
| Method proposed by  | ###### GREECE |  |  |
| Method commonly used in  | N. Greece |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Benthic Macroinvertebrates. A family based sensitivity score in relation to abundance (HBMWP) and the average score per taxon (HASPT) are estimated and then according to habitat richness an index and a 5 quality class interpretation is given. This classification is examined as to its precision with a discriminant classification method. Sites are chosen randomnly at a river basement level and before and after points sources of pollution. The 3-minute kick sweep method is used+ 1 minute hand net where vegetation occurs. Hydromorphology-habitat survey. The deviation between the habitats observed and reference habitat conditions is measured with four qualitative criteria and quality classes are given as to the modification of the habitats. The results are expressed with multi-factors maps with the help of GIS. Sites are chosen in a random strategic way trying to cover the 25% of each river channel with more stations downstream and at the high order channel streams (river basement). Physicochemical parameters. They are analysed at the same time with macroinvertebrates. Apart the habitat parameters (flow, discharge, substrate analysis, coordinates, land uses etc) pH, oC, turbitidy, TDS, conductivity, TSS, DO%, BOD. Redox, Cl, Alcalinity, Hardness, nutrients (NO2,NO3, NH4, PO4). Heavy metals, pesticides, hormones and substrate analysis where necessary. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| At the moment in the School of Biology a data base is built up with reference sites for benthic macroinvertebrates per River Basin. Additionally three different models are built up: a model like RIVPACS, a logistic model and a neural network model as well. These are prepared per river basin: the river basin of the lake PLASTIRA, that of the river ALIAKMON (totally Hellenic river) and that of the Vardar-Axios river (transboundary river). Database of best existing sites as to the babitats has been built up for 2 trounsboundary rivers (Aoos-Vjose and Vardar-Axios) belonging to two different ecoregions. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | No |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  |  |
| National Standard (yes/no) – if yes provide reference number |  |  |  |
| National method (yes/no) – if yes provide reference number and details |  |  |  |
| Published in literature (yes/no) – if yes provide reference details |  |  | Under preparation |
| Reference material |  |  |  |
| Technical reports about Vardar-Axios and Aoos-Vjose submitted to the Ministry of Environment (DAC Direction,E.E.). |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The above methods for the study of the proposed ecological quality elements of the WFD are consistent and allow assessment related to reference conditions with no or very few human pressure as well. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Prof.Maria Lazaridou |  |
| Expert e-mail address |  | mlazarid@bio.auth.gr |  |
| URL: |  | http://river.bio.auth.gr/labrivers |  |
| Telephone number |  | ++30-310-998373 |  |