IBMR - monitoring method

Monitoring WG 2.7 – 24/06/2002

| ##### Water Category |  | ##### River |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element |  | Aquatic flora – Macrophytes composition and abundance |  |
| Name of the method  |  | IBMR (Indice biologique macrophytes en rivière) |  |
| Method proposed by  |  | ###### France |  |
| Method commonly used in  |  | Experimented since 1996, revised in 2001 and 2002 following national implementations |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| IBMR aims to assess the trophic status of rivers by using a set of more than 200 taxa belonging to filamentous algae including some cyanobacteria clumps, bryophytes, lichens pteridophytes, phanerogames as well as very few filamentous bacteria and fungus. Sampling of aquatic macrophytes is carried out in at least 50 m rivers stretches by prospecting 2 different facies. The identification level is species excepted for algae and bacteria. Specific scores (from 0 to 20 according to the sensitivity to ammonia and phosphate) and stenoecy scores (from 0 to 3) have been attributed to each taxon. Abundance is also taken into account by using 5 covering classes. The index calculation is based on the Zelinka and Marvan formula. |  |  |  |
| This index has been first proposed for publication by Haury et al. in 1996. First national implementations (Haury and al., 1998) lead the French Macrophytes Scientific Interest Group to update both sampling methodology and scores in 2001. Field surveys in 2001 led to a last update in June 2002. This 2002 IBMR version is being examined for French standardisation. Note that this standard project is in compliance with CEN project standards for macrophytes surveys |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| - Haury J, Peltre M-C, Muller S, Tremolières M, Barbe J, Dutartre A, Guerlesquin M (1996). Des indices macrophytiques pour estimer la qualité des cours d'eau français : premières propositions. Ecologie 27(4) : 79-90. |  |  |  |
| - Haury J, Jaffre M, Dutartre A, Peltre M-C, Barbe J, Trémolières M, Guerlesquin M, Muller S (1998) Application de la méthode "milieux et végétaux aquatiques fixés" à 12 rivières françaises. Typologie floristique préliminaire. Annals. Limnol. 34(2): 129-138. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | No according to the Directive but reference conditions for trophy are included in the original data set |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | Yes, in compliance with CEN standard for macrophytes survey in rivers (in progress) |
| National Standard (yes/no) – if yes provide reference number |  |  | AFNOR in progress. |
| National method (yes/no) – if yes provide reference number and details |  |  | GIS Macrophytes des eaux continentales, 2002. Indice Biologique Macrophytes en Rivière (I.B.M.R.) : recommandations générales et indications pour une application 2002, février 2002, 25pp ###### Warning : score update in June 2002 |
| Published in literature (yes/no) – if yes provide reference details |  |  | No |
| Reference material |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| With expert system, the IBMR method is fairly consistant with WFD (reference indice values are included in the method as maximum value of 20 can be reached with different macrophytic assemblages according to the type of river), but the assessment is not completely related to reference condition with no or very few human pressure. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert | Jacques Haury Jean Prygiel |  |  |
| Expert e-mail address | Haury@roazhon.inra.fr j.prygiel@eau-artois-picardie.fr |  |  |
| URL: | http://www.eau-artois-picardie.fr |  |  |
| Telephone number | 33 (0) 2 23 48 55 39 33 3 27 99 90 21 |  |  |