Occurrence of river macrophytes - monitoring method

(fact sheet)

| ##### Water Category | River (watercourses in general) |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Macrophytes and phytobenthos– composition and frequency |  |  |
| Name of the method  | Occurrence of river macrophytes |  |  |
| Method proposed by  | ###### Sweden |  |  |
| Method commonly used in  | Denmark, Sweden accepted by the other Nordic countries |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| On each locality, i.e. a stream reach of 100 m, 6-10 transects are placed across the stream. Along each transect, 0.25 x 0.25 m quadrats are placed side by side from one bank to the other. If the river is deep with large non-vegetated areas, broken transects are made by starting from each side of the river, analysing to the outer limit of vegetation. In each quadrat the presence of bryophytes and vascular plant species is recorded. Observations are made by means of a viewing tube with clear perspex base. On each locality, relative frequencies of species in the quadrats are calculated (only quadrats with plants are considered). If the spatial extension of macrophytes is very sparse on a site, more quadrats should be obtained to ensure accurate calculations of frequencies. The number of quadrates is regulated by regulating the number of transects included in the sampling. The transects should preferentially be divided into depth zones, and frequencies calculated per depth zone. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | no |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | no |
| National Standard (yes/no) – if yes provide reference number |  |  | no |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes. SEPA.www.environ.se Manual Version 1:1:2002-04-10 |
| Published in literature (yes/no) – if yes provide reference details |  |  | no |
| Reference material |  |  |  |
| Biological Monitoring of Rivers and lakes. TemaNord 2001 :513. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Intense surveying of Swedisk watercourses is needed to get a data base of macrophytes as a tool for classification. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Berta Andersson |  |
| Expert e-mail address |  | Berta.Andersson@ma.slu.se |  |
| URL: |  |  |  |
| Telephone number |  |  |  |