CIS Working Group 2.7 Monitoring

The potential role of bioassays in meeting the monitoring needs of the Water Framework Directive.

The role of bioassays for use in helping to implement the Water Framework Directive was raised at a recent Working Group 2.7 meeting and the UK was asked to produce a paper for discussion on this issue.

# Bioassays

A bioassay is a tool which enables us to see if an environmental or waste sample has an effect on mortality, growth or reproduction of an organism (although other endpoints can be used). An example would be the exposure of water fleas (Daphnia) to river water to determine any effects on mortality, growth or reproduction. Bioassays are most commonly carried out on discreet spot samples taken from the environment back to a laboratory but they can also be conducted in-situ  (eg assessment of feeding rate of freshwater shrimps in-situ). Equally, they can be set-up to operate “on-line” (eg fish and invertebrate activity monitors such as those used to assess water quality on the Rhine). In the aquatic environment, bioassays can be conducted on both water samples and sediments.

Bioassays present an opportunity for a more holistic (and therefore more meaningful) way of assessing effects of environmental samples and wastes on ecosystems than is possible using chemical-based monitoring alone. They can provide predictions of environmental impacts whereas ecological community measures only determine impacts after they have occurred. Furthermore, they can be good diagnostic tools which enable the causes of poor ecological quality to be determined and traced back to source. The introduction of microscale/high-throughput laboratory based methodologies in recent years has enabled large numbers of samples to be tested at minimal cost, yet still ensuring the data generated is high quality and “fit for purpose”. Bioassays need not be any more difficult or costly to perform than either chemical or ecological community measures. As with all tools, bioassays have limitations – the ecological relevance of some observed responses may not be known; sampling induced artefacts may confound results; sample instability may preclude chronic laboratory testing and in-situ bioassays are vulnerable to damage. It is important to be aware of these limitations in planning studies and in interpreting the results. Bioassays should be viewed as an important “tool in the toolbox” for environmental management adding value and providing complementary information to that provided by the chemical and ecological community measures.

The use of bioassays in environmental management has a long international history particularly in North America. The use of bioassays for environmental assessment is also well established in European programmes such as the OSPAR Co-ordinated Environmental Monitoring Programme and in the UK National Marine Monitoring Programme. Bioassays have been used on a limited extent for effluent assessment and control by European member states. There application in this area has recently been reviewed by an OSPAR Whole Effluent Assessment International Working Group (OSPAR WEA IEG, 2000 – see references). Additionally, the Environment Agency for England & Wales in collaboration with others has recently completed an extensive literature review of the role, application and guidance for the use of bioassays in the monitoring and management of the water environment. The Agency has further developed a general framework for the use of bioassays for this purpose (Environment Agency, 2001 a & b).

The Water Framework Directive

To meet the requirements of the Directive, member states need to establish tools and techniques for monitoring the ecological and chemical status of surface waters, enabling identification of both damaging pollution and its causes and sources. This will be necessary for rivers, lakes, transitional and coastal waters. Three types of monitoring are specified in Annex 5 of the directive –

- Surveillance monitoring

- Operational monitoring

- Investigative monitoring

The principal objective of surveillance monitoring is to supplement and validate risk assessments undertaken to determine which water bodies could be at risk of failing to achieve key objectives like good status. Operational monitoring programmes are exclusively focused on establishing the status of those water bodies that, on the basis of the risk assessments and the surveillance monitoring programmes, are at risk of failing to meet the Directive’s environmental objectives. Investigative monitoring is required to ascertain the cause and effects of the problem, and help design appropriate management measures.

The following, recommendations are made on the specific role of bioassays for each of these types of monitoring. These are in line with recommendations made by the Scientific Committee for Toxicity, Ecotoxicity and the Environment set-up to support the development of the Water Framework Directive.

Surveillance Monitoring

Surveillance monitoring will be undertaken to :  -

- supplement and validate the procedures used to provide an impact assessment of waterbodies

- aid the efficient and effective design of future monitoring programmes

- assess long-term changes in natural conditions

- assess long-term changes resulting from widespread anthropogenic activity

Surveillance monitoring requires monitoring for all biological and general physico-chemical elements. Also, Priority List substances discharged into the river basin and other pollutants discharged in significant quantities should be measured. However, bioassays could be used to detect the presence of a wide range of pollutants more efficiently and effectively than speculatively attempting to analyse water samples for large numbers of individual pollutants.

Therefore, bioassays could be used : -

- in determining whether or not the risk assessments in relation to the discharge of chemical pollutants are correct – in particular to avoid an under protective outcome. (but only if the tests and the endpoints measured are comparable – it would not be appropriate to use acute lethal bioassays to validate risk assessments carried out to ensure protection against chronic sublethal effects)

- where the water body is heavily modified and therefore habitat constraints and not water quality constraints are restricting ecological status. In this case, bioassays would be used as a surrogate for measures of ecological status ie by indicating whether or not water quality could support a good diversity of aquatic life.

- where measures of ecological diversity with sufficient discriminatory powers to enable differences in status to be discerned have yet to be developed. In this case, bioassays would be used as a temporary surrogate for measures of ecological status until these measures become available.

- where there is a need to look for trends in biological quality (bioassays can be less variable than biological community measures).

Operational Monitoring

Operational monitoring will be undertaken to : -

- establish the status of river basins identified as being at risk of failing to meet their environmental objectives

- assess any changes in the status of such bodies resulting from the programme of measures used to reduce the impact of relevant pressures.

Bioassays can be used to show changes in receiving water quality which can be correlated with changes in relevant pressures (eg changes in the quality of effluents entering a river basin (point-source) or changes in the quantities of nutrients entering a river basin (point-source and/or diffuse)). These changes could be either a deterioration in water quality or an improvement in water quality. The bioassay response will tend to be quicker than that of the ecological diversity measure thus allowing prompt corrective action to be taken in the case of a deterioration in water quality or an early indication that corrective action had worked in the case of an improvement in water quality.

Where point-source inputs are involved, the same bioassay measure can be used to assess the quality and to subsequently link cause and effect in the receiving water ie predict likely impact). Furthermore, the bioassays can be used to predict the success of options for reducing the impact of point-source inputs thus ensuring good investment decisions.

Investigative Monitoring

Investigative monitoring will be undertaken to : -

- further investigate the reasons for exceedances in environmental quality standards where these are unknown.

- investigate the reasons for a water body not achieving its environmental objective where operational monitoring has not already been established.

- ascertain the magnitude and impacts of accidental pollution.

Bioassays can be used in a diagnostic way to help determine the reasons for restricted ecological status ie to separate out effects due to toxicity and nutrient effects from those due to other water quality problems (eg dissolved oxygen and suspended solids) or from other habitat constraints (eg heavily modified water bodies or a changed flow regime). Furthermore, bioassays can be used to effectively track back to the source of a toxicity problem in a river system and in wastewater drainage systems (toxicity tracking). Bioassays can be used to assess the biologically-active fraction of water or effluent samples after a series of physico-chemical manipulations of the sample designed to identify the contaminants causing the toxicity (toxicity identification evaluation). The use of bioassays for investigative monitoring is probably their most important role related to the requirements of the Directive.

Accidental pollution

The same diagnostic approach can be applied to help identify the source and nature of contaminants introduced into waterbodies as a result of accidental pollution.

Conclusions

Bioassays integrate the effects of all the contaminants (including unknown substances, some of which may be breakdown products) in environmental samples and wastewaters and take account of any additive and synergistic effects (although all species are not equally sensitive to all contaminants). They also provide a predictive capability that can help prevent environmental impact. The recent development of microscale/high-throughput methodologies has enabled large numbers of samples to be processed at minimal cost. Bioassays are viewed as important “tool in the toolbox” for water quality management worldwide.

This paper summarises the potential areas of monitoring where bioassays can contribute to the efficient management of the Directive’s monitoring programmes and we recommend that the Working Group consider including the bioassay in the Guidance.

References

OSPAR (2000), OSPAR Background document concerning the elaboration of programmes and measures relating to the whole effluent assessment. OSPAR Commission, 2000.

ISBN 0 946955 98 0

Environment Agency, 2002a. Role, Application and Guidance for Use of Bioassays in the Monitoring and Management of the Water Environment – Literature Review. R&D Technical Report E1-083/TR2. ISBN : 1 85 705 793 7

Environment Agency, 2002b. General Framework for the Use of Bioassays in the Monitoring and Management of the Water Environment. R&D Technical Report E1-083/TR1. ISBN 1 85 705 794 5

DT

9.7.02