Effect-directed identification procedures - monitoring method

(fact sheet)

| ##### Water Category | Coastal/Transitional water |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Contaminants |  |  |
| Name of the method  | Effect-directed identification procedures |  |  |
| Method proposed by  | ###### Netherlands |  |  |
| Method commonly used in  | Netherlands, Germany, UK, Belgium, Spain, Sweden |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| In a joint effort, ITLH and OSCM developand validate part of an instrument for integrated biological and chemical assessment of ecological risks of sediment. The instrument consists of three building blocks: in vivo bioassays, in vitro screening assays and a TIE (Toxicity Identification and Evaluation) tool. This TIE tool focuses primarily on the evaluation of the toxicity as observed in in vitro screening assays, and constitutes of a package of extraction, clean-up and fractionation techniques combined with a GC/MS and Database system (QPID). Currently, attention is mainly focused on identification of compounds having genotoxic and dioxin-type mechanisms. Analytical-chemical procedures are validated using a set of standard compounds with a broad range of physical-chemical properties. Procedures used were: Accelerated Solvent Extraction with dichloromethane, GPC-cleanup and solvent-exchange into ethylacetate for GC/MS or methanol or DMSO for toxicity testing. Toxicity was determined with the Mutatox bacterial genotoxicity test and the DR-CALUX assay. GC/MS chromatograms were processed using the AMDIS software, which statistically analyzes the chromatogram for masses which coelute. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Lukasewycz, M., E. Durhan (1992). Strategies for the identification of non-polar toxicants in aqeous environmental samples using toxicity-based fractionation and gas chromatography-mass spectrometry. J. Chromatogr. 580, 215-228. Ferñandez, P., M. Grifoll, A.M. Solanas, J.M. Bayona, and J. Albagès (1992). Bioassay-directed chemical analysis of genotoxic components in coastal sediments. Environ. Sci. Technol. 26(4), 817-829. Griffol, M., A.M. Solanas, and J.M. Bayona (1992). Bioassay-directed chemical characterization of genotoxic agents in the dissolved and particulate phases of the Besos and Llobregat rivers (Barcelona, Spain). Arch. Environ. Contam. Toxicol. 23, 19-25. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | Yes |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | No |
| National Standard (yes/no) – if yes provide reference number |  |  | No |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes, RIKZ method Standard Operating Procedure A 903, 904 and 905. |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes, see above |
| Reference material |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Consistent |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | J.H. Jorritsma |  |
| Expert e-mail address |  | J.Jorritsma@rikz.rws.minvenw.nl |  |
| URL: |  | www.rikz.nl |  |
| Telephone number |  | +3150 5331442 |  |