Fish populations (transitional waters) seine netting - monitoring method

(fact sheet)

| ##### Water Category | Transitional |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Composition and abundance of fish fauna |  |  |
| Name of the method  | Seine Netting |  |  |
| Method proposed by  | ###### UK |  |  |
| Method commonly used in  | UK |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| The seine net is loaded onto a boat with one end of the net held on the shore. During deployment (usually at low water slack), the boat sets the net out in a semi-circle, returning the other end of the net back to the shore. The two ends of the net are then hauled in towards the shore. The target organisms for this method are pelagic fish species. The technique is applied alongside beam trawling and kick sampling at the same sampling location to obtain information on the fish community present at the site. The fish captured in the seine net are identified to species level in the field, counted and measured (fork length). The fish captured are returned to the transitional water alive where possible. A standard method is used to derive measures of total number of species and total abundance. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| The measures of fish community structure are not compared to reference conditions at present. Research is ongoing on how measures derived using this method can be used for the purposes of the WFD. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | No |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | No |
| National Standard (yes/no) – if yes provide reference number |  |  | No |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes – Environment Agency National Marine Procedures Manual (Ref: D103). |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes |
| Reference material |  |  |  |
| Environment Agency – Tidal Thames Fisheries Survey, EA Thames Region. European Commission Report – FAIR CT961634: Commercial Fish & European Estuaries – Priorities for Management and Research. Elliott, M. & Hemingway, K. (ed) (2002). Fishes in Estuaries, Blackwell Science Ltd, London. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The method provides the means for estimating the composition and abundance of the pelagic component of fish communities in transitional waters. The method forms part of output of EA R&D Project E1-116 which is looking at developing a fish based classification scheme and monitoring strategy for Transitional Waters. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Steve Coates |  |
| Expert e-mail address |  | Steve.coates@environment-agency.gov.uk |  |
| URL: |  | www.environment-agency.gov.uk |  |
| Telephone number |  | 020 8310 5500 |  |