Fish populations (transitional waters) power station intake screens - monitoring method

(fact sheet)

| ##### Water Category | Transitional |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Composition and abundance of fish fauna |  |  |
| Name of the method  | Power Station Intake Screens |  |  |
| Method proposed by  | ###### UK |  |  |
| Method commonly used in  | UK |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Some power stations require large volumes of cooling water (e.g. West Thurrock on the Thames Estuary extracts up to 136 million litres per hour) and for this reason they are located on the coasts of transitional and coastal waters. Cooling water intakes of power stations located in transitional waters have been exploited to sample local fish communities. Screens are used to prevent fish and debris entering the cooling system and the fish entrained on these screens are used to obtain samples of fish populations. The technique has been used successfully to monitor the recovery of the fish populations in the Thames estuary. It produces estimates of fish species present and of relative abundance. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| The measures of fish community structure are not compared to reference conditions at present. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | No |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | No |
| National Standard (yes/no) – if yes provide reference number |  |  | No |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes |
| Reference material |  |  |  |
| Environment Agency – Tidal Thames Fisheries Survey, EA Thames Region. European Commission Report – FAIR CT961634: Commercial Fish & European Estuaries – Priorities for Management and Research. Thomas, M. in : Attrill, M.J. (ed) (1998). A rehabilitated estuarine ecosystem. The environment and ecology of the Thames Estuary, Kluwer Academic Publishers. Elliott, M. & Hemingway, K. (ed) (2002). Fishes in Estuaries, Blackwell Science Ltd, London. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The method provides the means for estimating the composition and abundance of the pelagic component of fish communities in transitional waters. The method forms part of output of EA R&D Project E1-116 which is looking at developing a fish based classification scheme and monitoring strategy for Transitional Waters. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Steve Coates |  |
| Expert e-mail address |  | Steve.coates@environment-agency.gov.uk |  |
| URL: |  | www.environment-agency.gov.uk |  |
| Telephone number |  | 020 8310 5500 |  |