REH - monitoring method

Monitoring WG 2.7 – 29/05/2002

| ##### Water Category | River |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Hydromorphology |  |  |
| Name of the method  | REH (habitat assessment network) |  |  |
| Method proposed by  | ###### France |  |  |
| Method commonly used in  | France |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| This method is based on the assessment by expert judgment of the level of alteration of fish habitat. The analysis scale is the ecologically homogeneous river reach. This assessment is performed trough a national network of 600 agents of the Conseil supérieur de la pêche in charge of the aquatic environment police. The following features are examined : discharge, channel morphology, channel and substrate diversity, banks, lateral and longitudinal continuity, floodplain and afferences. The human pressures are listed. The expert judgments are standardized by a national inquiry form and an assessment protocol. The features of environnement description, alteration and pressures are in adequacy with the WFD. A diagnostic on all the French streams is planned for December 2004. |  |  |  |
| The REH will be integrated into the physical SEQ in 2004. |  |  |  |
| The results of this method have been tested and validated in britanny by statistics relations with the fish metrics. The application of REH methodology outside an expert network should need special caution. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| - Conseil Supérieur de la Pêche, Agence de l’eau Loire Bretagne, 1996. Qualité écologique des cours d’eau de Bretagne, 67 p. |  |  |  |
| - Conseil Supérieur de la Pêche, Agence de l’eau Loire Bretagne, 1999. Qualité écologique des cours d’eau du bassin de la Maine, 101 p. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | <to be completed> |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | <to be completed> |
| National Standard (yes/no) – if yes provide reference number |  |  | <to be completed> |
| National method (yes/no) – if yes provide reference number and details |  |  | <to be completed> |
| Published in literature (yes/no) – if yes provide reference details |  |  | <to be completed> |
| Reference material |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Thibault Vigneron |  |
| Expert e-mail address |  | thibault.vigneron@csp.environnement.gouv.fr |  |
| URL: |  |  |  |
| Telephone number |  | 33 2 23 45 06 04 |  |