ECN aquatic macrophytes (lakes) - monitoring method

(fact sheet)

| ##### Water Category | Lakes |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Composition and abundance of aquatic flora |  |  |
| Name of the method  | Protocol for monitoring aquatic macrophytes at ECN lake sites |  |  |
| Method proposed by  | ###### UK |  |  |
| Method commonly used in  | UK |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| The aquatic macrophytes protocol is aimed at monitoring change in the aquatic macrophyte communities in lakes. The method generates estimates of the species present and their relative abundance and species zonation at selected sample points in lakes. |  |  |  |
| Sampling site location is undertaken in the context of the macrophytes resources of the lake and rerpresentative locations are defined. The protocol provides procedures for shoreline and deep water surveys. Shoreline sampling is undertaken in a 100 m stretch of shore which is divided into 10 m sections using canes or stakes. All species of emergent, floating and submerged macrophytes are recorded and percentage cover in each 10 m section estimated. For deep water sampling a transect from the shore to the edge of the phytolittoral zone is laid and samples from the bottom are taken using a combination of direct observations, grab and grapnel samples. |  |  |  |
| In addition to the macrophyte recording, the relevant site characeristics are also noted. |  |  |  |
| Sampling is undertaken every two years in July. The species compositions and estimates of percentage cover are used primarily for temporal comparisons at the same site. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| The macrophyte data generated are not compared to reference conditions. The prime purpose of the ECN monitoring is temporal change over a long time-scale. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | No |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | No |
| National Standard (yes/no) – if yes provide reference number |  |  | No (However, the method is based on a National Standard method (SCA, 1987)) |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes – for ECN monitoring network (FMA protocol – see reference material) |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes |
| Reference material |  |  |  |
| Sykes, JM., Lane, AMJ. and George, G. (Eds) (1999). The United Kingdom Environmental Change Network: protocols for standard measurements at freshwater sites. Institute of Terrestrial Ecology, Huntingdon. (ISBN: 1 870393 49 X) |  |  |  |
| Standing Committee of Analysts (1987).Methods for the use of aquatic macrophytes for assesing water quality 1985-6. HMSO, London. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The sampling methodology provides a suitable basis generating the required measures for the WFD (species composition and abundance (percentage cover)). |  |  |  |
| The measures are not related to reference conditions and there is no classification scheme relating to quality status. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Alastair Ferguson |  |
| Expert e-mail address |  | Alastair.ferguson@environment-agency.gov.uk |  |
| URL: |  | www.environment-agency.gov.uk |  |
| Telephone number |  | 01454 878849 |  |