Environmental Quality Criteria - Benthic fauna - lakes

(fact sheet)

| ##### Water Category | Lakes – profundal zone |  |
| --- | --- | --- |
| ##### Water Framework Directive Quality element | Benthic invertebrate fauna – composition and diversity (using indices) |  |
| Name of the method  | Environmental Quality Criteria – Benthic fauna - lakes |  |
| Method proposed by  | ###### Sweden |  |
| Method commonly used in  | Sweden |  |
| Short description of the method (approx. 10 lines): |  |  |
| For lakes (profundal zone), benthic fauna is sampled using five standardised Ekman grabs. All sorting is done in the laboratory and identification is to a predetermined level (using 517 taxa). For assessment two indices (the BQI index and the O/C index) is used to evaluate the human stress (e.g., eutrophication) on lakes. |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |
| Best available sites for six ecoregions (types). Class boundaries for the assessment system is based on statistical breakdowns, where class ‘high’ and ‘bad’ are delimited by the 90th and 10th percentile. The classification is based on the EC white paper by Nixon et al. 1996. The references are spatiallyand temporally based using data from the Swedish National lake monitoring program. |  |  |
| Reference conditions compliant with WFD (yes/no) |  | Yes |
| Status of method |  |  |
| International Standard (yes/no) – if yes provide reference number |  | No |
| National Standard (yes/no) – if yes provide reference number |  | Yes, SS 028190 For sampling |
| National method (yes/no) – if yes provide reference number and details |  | Yes |
| Published in literature (yes/no) – if yes provide reference details |  | The assessment system is published in the SEPA report 5050. |
| Reference material |  |  |
| Environmental Quality Criteria for Lakes and Watercourses. Report 5050. Swedish Environmental Protection Agency (for assessment). Swedish EPA Environmental Monitoring Handbook. (www.naturvardsverket.se in Swedish) (for sampling). |  |  |
| Applicability of the method to WFD (Expert’s assessment of method): |  |  |
| The system is compliant with the WFD since it contains both an Ecological Quality Ratio (EQR), a reference condition and is based on an ecoregional typology. |  |  |

| Contact details |  |
| --- | --- |
| Name of contact/expert | Leonard Sandin |
| Expert e-mail address | Leonard.sandin@ma.slu.se |
| URL: | www.ma.slu.se |
| Telephone number | +46-(0)18-673813 |

29/05/2002