Chlorophyll a - monitoring method

(fact sheet)

| ##### Water Category | Lakes |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Phytoplankton |  |  |
| Name of the method  | Determination of chlorophyll a, spectrophotometric determination in methanol extract |  |  |
| Method proposed by  | ###### Norway |  |  |
| Method commonly used in  | Norway, Sweden, Denmark and possibly many other countries |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Sample from epilimnion or metalimnion is filtered on glassfiberfilter (GF/C). The volume filtered depend on the algal biomass present in the lake, and varies normally from 0,1-1 L. After filtration the filters should be frozen untill analysis. Chlorophyll a on the filter is extracted in methanol overnight. After sentrifugation of extract the absorbance is measured in spectrophotometer at 750 and 665 nm, and chlorophyll concentration calculated with a simple formula. |  |  |  |
| The results are used to assess the ecological status along the eutrophication gradient. Five status classes with absolute values are given (<2, 2-4, 4-8, 8-20 and > 20 µg /L). |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): e.g. Database of best existing sites for range of water body types |  |  |  |
| Deviation from reference conditions are not included, but can easily be elaborated when reference conditions for chlorophyll in different lake types are identified. Database of best existing sites for a range of water body types will be available at the Norwegian Institute for Water Research and at the State Pollution Control Authority in Norway, as soon as the typology system for lakes are developed. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | no, but can easily be made compliant when ref-cond. for different lake types are identified |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | no, but intercalibration is presently performed |
| National Standard (yes/no) – if yes provide reference number |  |  | yes, Norwegian Standard NS 4767 (analysis) Norwegian State Pollution Control Authority 97:04: Classification of environmental quality in freshwater (assessment of ecological status) |
| National method (yes/no) – if yes provide reference number and details |  |  | yes, Norwegian Standard NS 4767, 1983 (in Norwegian only). The standard is equivalent to Danish Standard DS 296 and Swedish Standard SS 02 81 70. Norwegian State Pollution Control Authority 1997: Classification of environmental quality in freshwater (in Norwegian only): Guidance 97:04. ISBN: 82-7655-368-0 |
| Published in literature (yes/no) – if yes provide reference details |  |  | Yes (see below) |
| Reference material |  |  |  |
| Analysis: Richards, F.A. and T.G: Thompson 1952. The estimation and characterization of plankton populations by pigment analysis. II. A spectrophotometric method for estimation of plankton pigments. J. mar. Res. 11: 156-172. Holm-Hansen, O. and B. Riemann 1978. Chlorophyll a determination: Improvements in methodology. Oikos 30: 438-447. Marker, A.H.F., Nusch, E.A., Rai, H. and B. Riemann 1980. The measurement of photosynthetic pigments in freshwaters and standardization of methods: Conclusions and recommendations. Arch. Hydrobiol., Beih. Ergebn. Limnol., 14: 91-106. |  |  |  |
| Assessment of ecological status: State Pollution Control Authority 1997. Classification of environmental quality in freshwater (in Norwegian only). SFT-guidance 97:04. 31 pp. ISBN: 82-7655-368-0. |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Highly applicable for assessment of phytoplankton biomass. Easy and relatively cheap method. In member states with more highly eutrophic lakes the borders between status classes may differ from those in the Norwegian system. Deviation from reference conditions can be estimated as soon as reference conditions for different lake types are identified. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Kai Soerensen (analysis), Anne Lyche Solheim (assessment) |  |
| Expert e-mail address |  | kai.soerensen@niva.no anne.lyche@niva.no |  |
| URL: |  | www.niva.no |  |
| Telephone number |  | 47-22185275 or 47-22185229 |  |