- monitoring method

(fact sheet)

| ##### Water Category | Rivers, Transitional Waters and Coastal Waters |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Chemical and physico-chemical |  |  |
| Name of the method  | Guidance on Input Trend Assessment and the Adjustment of Loads |  |  |
| Method proposed by  | ###### Netherlands as OSPAR Lead Country |  |  |
| Method commonly used in  | Spain, Portugal, France, United Kingdom, Ireland, Iceland, Norway, Sweden, Finland, Denmark, Germany, the Netherlands, Belgium, Luxembourg, Switzerland, European Community |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Within OSPAR, the objective of trend assessment of inputs is to identify and quantify any significant pattern of change in inputs in order to support the mission of OSPAR under the relevant OSPAR Strategies and the OSPAR Joint Assessment and Monitoring Programme (JAMP). |  |  |  |
| While there is a need to consider both actual and adjusted inputs in any overall assessments, adjusted inputs can help in determining the effectiveness of measures relating to sources within catchment areas where flow variation has masked any underlying pattern of change. |  |  |  |
| The conceptual Approach for Adjustment and Trend Analysis consists of five steps: - Provision of raw data, including necessary initiating processing (dealing with missing values, outliers, values below limit of detection) - Adjustment of loads to compensate for the effect of irregular fluctuations induced by varying flow rates, precipitation and SPM - Aggregation of the adjusted loads in order to consider both actual and adjusted inputs in any assessments - Trend assessment to test objectively whether there is a meaningful systematic change in the time series - Power analysis to quantify the detectable trend or to evaluate the monitoering programme |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  |  |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  | No (the intention is to have it agreed in 2003) |
| National Standard (yes/no) – if yes provide reference number |  |  | No |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes http://statserver.candiensten.nl/rikz/ |
| Published in literature (yes/no) – if yes provide reference details |  |  | No |
| Reference material |  |  |  |
| Not relevant |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| The method contains guidance on load adjustment and trend assessment. The load adjustment is applicable for the rivers and substances for which the basic information is readily available. For other rivers and/or substances, there would be either merit in establishing simpler methods of adjustment or the basic information has to be derived. |  |  |  |
| The trend assessment method is in general applicable to other series of annual indices about water quality (which was underpinned by Dutch/German research on a variety time series). The main choice for a method can be made on the length of the time series. A strong point is the possibility to incorporate knowledge about the variation of the annual index. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Richard Duin (statistical) Andrew Osborne (general) |  |
| Expert e-mail address |  | r.n.m.duin@rikz.rws.minvenw.nl andrew.osborne@defra.gsi.gov.uk |  |
| URL: |  | www.waterland.net/rikz/osparwg |  |
| Telephone number |  | +31 70 311 4214 +44 20 7944 5313 |  |