Light attenuation - assessment method

(fact sheet)

| ##### Water Category | Coastal waters / Baltic Sea |  |  |
| --- | --- | --- | --- |
| ##### Water Framework Directive Quality Element | Chemical and physico-chemical elements supporting the biological elements |  |  |
| Name of the method  | Light attenuation |  |  |
| Method proposed by  | ###### Sweden |  |  |
| Method commonly used in  | Sweden |  |  |
| Short description of the method (approx. 10 lines): |  |  |  |
| Sampling and laboratory : see HELCOM |  |  |  |
| Assessment: Transparency may also be affected by the presence of other substances, such as humus and sludge. Large additions of such materials may explain why there is no clear connection between plankton concentrations and transparency in the Bothnian Bay and the Bothnian Sea. Therefore, transparency is recommended as an indirect measure of eutrophication only for the Baltic proper and the Skagerrak/Kattegat. All of the measurements presented below refer to transparency in those areas during the month of August. Assessment of current conditions : The classification is based on transparency measured along the Swedish coast during 1988–90. The class limits have been set so that each class includes one-fifth of the measured values. |  |  |  |
| Deviations from reference values : The quotient between measured transparency and reference value is used in this context to indicate to which extent the transparency measured varies from natural, pristine conditions. A quotient of 1 (measured transparency/reference value) is used as the boundary between classes 1 and 2. The boundary between classes 4 and 5 has been set so that class 5 includes ten percent of the levels measured during 1988-90 in waters along the Swedish coast. In general, the transparency levels of class 5 indicate pollution from a local point source. |  |  |  |
| Reference basis of method and comparison (approx. 10 lines): |  |  |  |
| Reference values are estimates of the transparency to be expected in waters that have not been significantly affected by human activities. Separate reference values have been provided for each type of area in the Baltic proper and the Skagerrak/Kattegat indicated. The reference values for the Baltic proper are based on historical data and constitute averages for the period around 1950. For Skagerrak/Kattegat, they are instead based on averages during 1979–93; those values have been adjusted upwards on the basis of currently available information. |  |  |  |
| Reference conditions compliant with WFD (yes/no) |  |  | Yes, but need further development. |
| Status of method |  |  |  |
| International Standard (yes/no) – if yes provide reference number |  |  |  |
| National Standard (yes/no) – if yes provide reference number |  |  |  |
| National method (yes/no) – if yes provide reference number and details |  |  | Yes |
| Published in literature (yes/no) – if yes provide reference details |  |  | Swedish Environmental Protection Agency 2000. Environmental Qality Criteria – Coasts and Seas. Report 5052. http://www.internat.naturvardsverket.se/index.php3, Legislation/guidelines, Environmental Quality Criteria, Coasts and Seas |
| Reference material |  |  |  |
| Applicability of the method to WFD (Expert’s assessment of method) |  |  |  |
| Yes. But development needed i.e. for setting class boundaries between status classes and adjusting typology. |  |  |  |
| Contact details |  |  |  |
| Name of contact/expert |  | Tove Lundeberg |  |
| Expert e-mail address |  | tove.lundeberg@naturvardsverket.se |  |
| URL: |  |  |  |
| Telephone number |  |  |  |